Richard Bi, Kevin Liu, Bryan Zhang: Chig Bungus

Checkpoint 1 Progress Report

Functionalities Implemented

- Instruction fetch module: Everyone

  - Similar to if_stage from mp_pipeline, except instruction are pushed into the queue once it’s read from memory/cache (currently magic memory)

- Top level module (CPU): Everyone

  - Contains global signals and instantiates instruction fetch and instruction queue instances

- Parameterizable queue: Everyone

  - Queue parameterizable by depth and width, has data I/O signals as well as push/pop and clock and reset. Pushes are done via sequential logic while pops are read combinationally (responds on the same clock cycle as the pop command)

- Overall Diagram: Everyone

  - Rough overview of planned design. Currently we are planning to add Superscalar capabilities and a store buffer that looks like it will work well with our design.

Testing Strategy

- Push w/o pop, push while popping

  - Push without popping is tested simply with top_tb instantiating a CPU and letting run on a memory file generated from mp_pipeline’s test code. Without any signals trying pop the queue, the queue will fill up and the CPU will stall forever

  - Push while popping is done with a pop signal from the testbench manually controlling the pop signal of the instruction in the CPU, once an instruction has been pushed into the queue, we turn on the pop signal. Then we let the queue fill up and then set the pop signal again to test pushing while popping while the queue is full

- Pop w/o push

  - We connected the push signal of the queue with a manually controlled signal in top_tb to set it to 0 entirely, then let the CPU run on a test program. This tests if popping without pushing works

Timing: 0.083 slack

Roadmap: will likely be a VSCode live share session

- Multiplier: Figure out how the multiplier works, especially its timing and I/O

- Adder

- Reservation stations

- Physical register file

- Rename/dispatch stage

- RAT/free list/RRF/ROB, commit logic